# RASBERRY PI PICO-TECHNICAL RESEARCH

| S.No. | Specification | Verified value |  |
| --- | --- | --- | --- |
| 1 | Board name | Raspberry Pi Pico |  |
| 2 | Microcontroller | RP2040 |  |
| 3 | Processor architecture | Dual-core ARM Cortex-M0+ |  |
| 4 | Maximum clock speed | 133 MHz |  |
| 5 | Flash memory | 2 MB on the standard original Pico board |  |
| 6 | SRAM | 264 KB |  |
| 7 | GPIO pins | 26 multifunction GPIO pins exposed |  |
| 8 | ADC | 12-bit ADC, 3 external analog inputs available |  |
| 9 | Built-in Wi-Fi | No |  |
| 10 | Built-in Bluetooth | No |  |
| 11 | Communication interfaces | SPI, I²C, UART, PWM |  |
| 12 | Programming languages | C/C++, MicroPython |  |
| 13 | Development software | Raspberry Pi Pico SDK, Thonny |  |
| 14 | Operating voltage | 3.3 V GPIO logic |  |
| 15 | Official source | Raspberry Pi Pico documentation |  |

Official references:

- Raspberry Pi Pico Official Documentation

- RP2040 Datasheet

Important: These specifications refer to the original Raspberry Pi Pico, not the Pico W. The Pico W has wireless connectivity, so we must not mix its specifications with the original Pico.

Research note: The original Raspberry Pi Pico is based on the RP2040 microcontroller, which features two ARM Cortex-M0+ cores. Unlike the Pico W, the original Pico does not have built-in Wi-Fi or Bluetooth. It is suitable for embedded control and sensor-based projects.