# STM32 Blue Pill — Technical Research

| S.No. | Specification | Verified value |
| --- | --- | --- |
| 1 | Board name | STM32 Blue Pill |
| 2 | Microcontroller | STM32F103C8T6 |
| 3 | Processor architecture | 32-bit ARM Cortex-M3 |
| 4 | Maximum clock speed | 72 MHz |
| 5 | Flash memory | 64 KB |
| 6 | SRAM | 20 KB |
| 7 | GPIO pins | Up to 37 I/O pins for the STM32F103C8 device |
| 8 | ADC | Two 12-bit ADCs |
| 9 | Built-in Wi-Fi | No |
| 10 | Built-in Bluetooth | No |
| 11 | Communication interfaces | SPI, I²C, USART, USB, CAN |
| 12 | Programming languages | C/C++ commonly used |
| 13 | Development software | STM32CubeIDE; other compatible tools are available |
| 14 | Operating voltage | 2.0–3.6 V for the microcontroller |
| 15 | Official source | STMicroelectronics datasheet |

Official sources:

- STM32F103C8 Official Product Page

- STM32F103C8 Datasheet (PDF)

These specifications are for the STM32F103C8 microcontroller. The exact pin availability and other board-level features can vary with the Blue Pill board design.

Research note: The STM32 Blue Pill is a low-cost development board built around the STM32F103C8T6 microcontroller. Unlike the ESP32, the standard Blue Pill does not provide built-in Wi-Fi or Bluetooth, so wireless connectivity generally requires additional hardware.